Current Project Specifications

Communication & Network

WiFi Connection: Dual-mode with fallback to AP mode

Client mode: Connects to specified SSID

AP mode: Creates "ESP32-Rover" network if client mode fails

Standard Web Server optimized for speed. Web-sockets may be used.

Verified with Firefox and Silk browsers

Serial monitor output showing network ID and IP address on WiFi connect.

Camera System

FPV streaming with configurable quality

Auto Quality Adjustment.

Orientation Control: Normal, Mirror, Flip, Mirror+Flip options

Video for best at network quality.

Web Interface

Status Indicators on the first row

WiFi connection status

WiFi signal strength (RSSI)

IP address

Board temperature in Fahrenheit .

Battery levels in volts.

SD card status

Next row: Full width streaming video.

Orientation selection

Next row:

On the right,Virtual Joystick: Touch-enabled control for movement. Return to center when released.

Map keyboard to W-forward, S-reverse, A-left, D-right.

Under direction joystick, Adjustable speed limit slider (0-255).

Next row

Camera LED brightness slider.

Next row

switch/buttons for selecting mode. Manual for remote joystick operation, autonomous roving. stationary trail motion sensor camera with recording to SD card .

with error capture for everything.

At the bottom, web serial monitor box. For esp32-cam and UNO.

Include documentation in source.

Use united states of America units of measure.

Include ASCII wiring diagram in sketch or separate tab.

Include authors for libraries and version number. URL for the library if you have it.

Include AI app (you) as contributing author with model used.

Use list of available hardware in file inventory.xlsx.

The file. Esp32-cam.txt has the know attributes of the esp32-cam i have.

The file atmega328.txt has the know attributes of the uno module i have.

Board information may be faulty.

If there is a existing module that would make the project so much better, let me know.

Existing project is at d:\projects\fpv_rover.

Optimize project.

Optimize wiring chart.

The current chassis have is 8 inches square.

I have 5 hc-sr04 mounted on the chassis.

Center front, pointing straight ahead.

Left front, 45 degrees left of center.

Right front, 45 degrees right of front.

Left, 90 degrees left of front.

Right, 90 right of front.

I have 4 hw-201 located between the hc-sr04 pointed down.

The chassis has a compont mounting web 11 inches by 7 inches.

Front and back is on the 11 inch sides.

I would like an optimized component placement for optimal wiring.

Hw-sr04 and hw-201 are not mounted on the web.

Also allow for a 2x1.5 inch 5v power bus.

Place the uno and esp32-cam usb connectors at the front or back.

Allow for a toggle switch on the 9 and 12 volt batteries.